1️⃣ 2:1 MUX

module mux_2to1(input a, b, sel, output y);

assign y = sel ? b : a;

endmodule

mux_2to1_tb.v

module mux_2to1_tb;

reg a, b, sel;

wire y;

mux_2to1 uut(a, b, sel, y);

initial begin

$display("A B SEL | Y");

$monitor("%b %b  %b  | %b", a, b, sel, y);

a=0; b=1; sel=0;

#5 sel=1;

#5 a=1; b=0; sel=0;

#5 sel=1;

#5 $finish;

end

endmodule

2️⃣ 1:2 DEMUX

module demux_1to2(input d, sel, output y0, y1);

assign y0 = (~sel) & d;

assign y1 = sel & d;

endmodule

module demux_1to2_tb;

reg d, sel;

wire y0, y1;

demux_1to2 uut(d, sel, y0, y1);

initial begin

$display("D SEL | Y0 Y1");

$monitor("%b  %b  | %b  %b", d, sel, y0, y1);

d=1; sel=0;

#5 sel=1;

#5 d=0;

#5 $finish;

end

endmodule

3️⃣ 2:4 DECODER

module decoder_2to4(input [1:0] a, output reg [3:0] y);

always @(*) begin

y = 4'b0000;

case(a)

2'b00: y = 4'b0001;

2'b01: y = 4'b0010;

2'b10: y = 4'b0100;

2'b11: y = 4'b1000;

endcase

end

endmodule

module decoder_2to4_tb;

reg [1:0] a;

wire [3:0] y;

decoder_2to4 uut(a, y);

initial begin

$display("A | Y");

$monitor("%b | %b", a, y);

a=0; #5 a=1; #5 a=2; #5 a=3;

#5 $finish;

end

endmodule

4️⃣ 4:2 ENCODER

module encoder_4to2(input [3:0] d, output reg [1:0] y);

always @(*) begin

case(d)

4'b0001: y = 2'b00;

4'b0010: y = 2'b01;

4'b0100: y = 2'b10;

4'b1000: y = 2'b11;

default: y = 2'bxx;

endcase

end

endmodule

encoder_4to2_tb.v

module encoder_4to2_tb;

reg [3:0] d;

wire [1:0] y;

encoder_4to2 uut(d, y);

initial begin

$display("D | Y");

$monitor("%b | %b", d, y);

d=4'b0001; #5

d=4'b0010; #5

d=4'b0100; #5

d=4'b1000; #5

$finish;

end

endmodule

5️⃣ HALF ADDER

module half_adder(input a, b, output sum, carry);

assign sum = a ^ b;

assign carry = a & b;

endmodule

half_adder_tb.v

module half_adder_tb;

reg a, b;

wire sum, carry;

half_adder uut(a, b, sum, carry);

initial begin

$display("A B | SUM CARRY");

$monitor("%b %b |  %b    %b", a, b, sum, carry);

a=0; b=0;

#5 a=0; b=1;

#5 a=1; b=0;

#5 a=1; b=1;

#5 $finish;

end

endmodule

6️⃣ FULL ADDER

module full_adder(input a, b, cin, output sum, carry);

assign sum = a ^ b ^ cin;

assign carry = (a & b) | (b & cin) | (a & cin);

endmodule

full_adder_tb.v

module full_adder_tb;

reg a, b, cin;

wire sum, carry;

full_adder uut(a, b, cin, sum, carry);

initial begin

$display("A B CIN | SUM CARRY");

$monitor("%b %b  %b  |  %b    %b", a, b, cin, sum, carry);

a=0; b=0; cin=0;

#5 a=0; b=1; cin=0;

#5 a=1; b=0; cin=1;

#5 a=1; b=1; cin=1;

#5 $finish;

end

endmodule

7) 1-BIT COMPARATOR

module comparator_1bit(input a, b, output gt, lt, eq);

assign gt = a & ~b;

assign lt = ~a & b;

assign eq = ~(a ^ b);

endmodule

comparator_1bit_tb.v

module comparator_1bit_tb;

reg a, b;

wire gt, lt, eq;

comparator_1bit uut(a, b, gt, lt, eq);

initial begin

$display("A B | GT LT EQ");

$monitor("%b %b |  %b  %b  %b", a, b, gt, lt, eq);

a=0; b=0;

#5 a=0; b=1;

#5 a=1; b=0;

#5 a=1; b=1;

#5 $finish;

end

endmodule

8) 4-BIT COMPARATOR

module comparator_4bit(

input [3:0] a, b,

output gt, lt, eq

);

assign gt = (a > b);

assign lt = (a < b);

assign eq = (a == b);

endmodule

comparator_4bit_tb.v

module comparator_4bit_tb;

reg [3:0] a, b;

wire gt, lt, eq;

comparator_4bit uut(a, b, gt, lt, eq);

initial begin

$display("A B | GT LT EQ");

$monitor("%b %b |  %b  %b  %b", a, b, gt, lt, eq);

a=4'd5; b=4'd3;

#5 a=4'd2; b=4'd7;

#5 a=4'd9; b=4'd9;

#5 $finish;

end

endmodule

9)MAGNITUDE COMPARATOR (GENERIC)

magnitude_comparator.v

module magnitude_comparator #(parameter N=4)(

input [N-1:0] a, b,

output a_gt_b, a_lt_b, a_eq_b

);

assign a_gt_b = (a > b);

assign a_lt_b = (a < b);

assign a_eq_b = (a == b);

endmodule

magnitude_comparator_tb.v

module magnitude_comparator_tb;

reg [3:0] a, b;

wire gt, lt, eq;

magnitude_comparator uut(a, b, gt, lt, eq);

initial begin

$display("A B | GT LT EQ");

$monitor("%b %b |  %b  %b  %b", a, b, gt, lt, eq);

a=8; b=4;

#5 a=3; b=6;

#5 a=5; b=5;

#5 $finish;

end

endmodule

10) HALF SUBTRACTOR

module half_subtractor(input a, b, output diff, borrow);

assign diff = a ^ b;

assign borrow = ~a & b;

endmodule

half_subtractor_tb.v

module half_subtractor_tb;

reg a, b;

wire diff, borrow;

half_subtractor uut(a, b, diff, borrow);

initial begin

$display("A B | DIFF BORROW");

$monitor("%b %b |  %b     %b", a, b, diff, borrow);

a=0; b=0;

#5 a=0; b=1;

#5 a=1; b=0;

#5 a=1; b=1;

#5 $finish;

end

endmodule

11) FULL SUBTRACTOR

module full_subtractor(input a, b, bin, output diff, borrow);

assign diff = a ^ b ^ bin;

assign borrow = (~a & b) | (~a & bin) | (b & bin);

endmodule

full_subtractor_tb.v

module full_subtractor_tb;

reg a, b, bin;

wire diff, borrow;

full_subtractor uut(a, b, bin, diff, borrow);

initial begin

$display("A B BIN | DIFF BORROW");

$monitor("%b %b  %b  |  %b     %b", a, b, bin, diff, borrow);

a=0; b=0; bin=0;

#5 a=0; b=1; bin=0;

#5 a=1; b=0; bin=1;

#5 a=1; b=1; bin=1;

#5 $finish;

end

endmodule